Simple 16-Bit CPU

Hardware Design Specification

Version 1.0

Implemented in Verilog HDL

# 1. Overview

This document specifies the architecture, instruction set, control pins, and internal operation of a simple 16-bit CPU implemented in Verilog HDL. The CPU features a register file with 8 general-purpose 16-bit registers, a pipelined datapath with arithmetic/logic unit, barrel shifter, and a finite state machine (FSM) controller. It executes a reduced instruction set supporting data movement, arithmetic, comparison, and logical operations.

## 1.1 Top-Level Block Diagram

The CPU is composed of four major functional blocks that communicate via internal control and data buses:

| Block | Module Name | Responsibility |
| --- | --- | --- |
| Instruction Register | register_load | Latches the 16-bit instruction from the external input bus on the load signal |
| Instruction Decoder | instruction_decoder | Decodes opcode, operand fields, and generates sign-extended immediates |
| Datapath | datapath | Register file, ALU, shifter, and intermediate flip-flops |
| FSM Controller | CPU_FSM | Sequences multi-cycle operations and generates all control signals |

# 2. Top-Level I/O Pin Reference

The following table describes every pin of the top-level cpu module:

| Pin | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1-bit | Input | System clock. All sequential logic is triggered on the rising edge. |
| reset | 1-bit | Input | Synchronous active-high reset. Returns the FSM to the S_WAIT state on the next rising clock edge. |
| s | 1-bit | Input | Start signal. When asserted in the S_WAIT state, causes the FSM to decode and begin executing the loaded instruction. |
| load | 1-bit | Input | Instruction load enable. When high, the 16-bit value present on in[] is latched into the instruction register on the next rising clock edge. |
| in | 16-bit | Input | Instruction / data input bus. The 16-bit instruction word to be executed must be presented on this bus before asserting load. |
| out | 16-bit | Output | Datapath result output. Reflects the value in the C flip-flop (result of the last completed ALU/shift operation). |
| N | 1-bit | Output | Negative flag. Set when the ALU result is negative (MSB = 1). Registered by the status flip-flop when loads is asserted. |
| V | 1-bit | Output | Overflow flag. Set when a signed arithmetic overflow is detected. Registered by the status flip-flop when loads is asserted. |
| Z | 1-bit | Output | Zero flag. Set when the ALU result is all-zero. Registered by the status flip-flop when loads is asserted. |
| w | 1-bit | Output | Wait indicator. Asserted (logic 1) when the CPU is idle in S_WAIT, waiting for the next instruction. |

# 3. Instruction Set Architecture

## 3.1 Instruction Word Format

All instructions are encoded as a fixed 16-bit word with the following general field layout:

| Bits [15:13] | Bits [12:11] | Bits [10:8] | Bits [7:5] | Bits [4:3] | Bits [2:0] |
| --- | --- | --- | --- | --- | --- |
| opcode (3) | op (2) | Rn (3) | Rd (3) | sh_op (2) | Rm (3) |

For immediate-type instructions, bits [7:0] carry an 8-bit immediate value (imm8) and bits [4:0] carry a 5-bit immediate (imm5). The decoder sign-extends both fields to 16 bits.

| Field | Bits | Description |
| --- | --- | --- |
| opcode | [15:13] | 3-bit primary opcode selecting the instruction class. |
| op | [12:11] | 2-bit secondary opcode refining the operation within a class. |
| Rn | [10:8] | First source / destination register specifier (3 bits -> R0–R7). |
| Rd | [7:5] | Destination register specifier (3 bits -> R0–R7). |
| sh_op | [4:3] | Shift operation applied to the Rm source before the ALU. |
| Rm | [2:0] | Second source register specifier (3 bits -> R0–R7). |
| imm8 | [7:0] | 8-bit unsigned immediate. Sign-extended to 16 bits (sximm8). |
| imm5 | [4:0] | 5-bit unsigned immediate. Sign-extended to 16 bits (sximm5). |

## 3.2 Instruction Descriptions

### MOV Rn, #<imm8>  —  Move Immediate

| Field | Value |
| --- | --- |
| Encoding | opcode = 3'b110, op = 2'b10 |
| Syntax | MOV Rn, #<imm8> |
| Operation | Rn <- sign_extend(imm8) |
| Flags Updated | None |
| Cycles | 2 (S_MOVim8, return to S_WAIT) |

Loads a signed 8-bit immediate value, sign-extended to 16 bits, directly into register Rn. The decoder generates sximm8 = {{8{in[7]}}, in[7:0]}. The FSM selects vsel = 4'b0100 to route sximm8 through the data_in multiplexer and asserts write with nsel = 3'b001 (selecting Rn).

### MOV Rd, Rm{,<sh_op>}  —  Move Register

| Field | Value |
| --- | --- |
| Encoding | opcode = 3'b110, op = 2'b00 |
| Syntax | MOV Rd, Rm{, sh_op} |
| Operation | Rd <- shift(Rm, sh_op) |
| Flags Updated | None |
| Cycles | 3 (S_GETB -> S_GETC -> S_MOVREG) |

Copies the shifted value of Rm into Rd. In S_GETB the FSM sets nsel = 3'b100 to read Rm and loadb = 1 to latch it into flip-flop B. In S_GETC asel = 1 forces Ain to zero and loadc = 1 stores the barrel-shifted result in flip-flop C. In S_MOVREG write = 1 with nsel = 3'b010 writes C to Rd. vsel = 4'b0001 selects the C register output throughout the write-back.

### ADD Rd, Rn, Rm{,<sh_op>}  —  Add

| Field | Value |
| --- | --- |
| Encoding | opcode = 3'b101, op = 2'b00 |
| Syntax | ADD Rd, Rn, Rm{, sh_op} |
| Operation | Rd <- Rn + shift(Rm, sh_op) |
| Flags Updated | None (loads not asserted) |
| Cycles | 4 (S_GETA -> S_GETB -> S_GETC -> S_MOVREG) |

Adds register Rn to the optionally shifted value of Rm, storing the result in Rd. S_GETA reads Rn (nsel = 3'b001) into flip-flop A (loada = 1). S_GETB reads Rm (nsel = 3'b100) into flip-flop B (loadb = 1). S_GETC performs the ALU add (ALUop = 2'b00, asel = 0, bsel = 0) and captures the result in C (loadc = 1). S_MOVREG writes C to Rd.

### CMP Rn, Rm{,<sh_op>}  —  Compare

| Field | Value |
| --- | --- |
| Encoding | opcode = 3'b101, op = 2'b01 |
| Syntax | CMP Rn, Rm{, sh_op} |
| Operation | temp <- Rn - shift(Rm, sh_op); update N, V, Z |
| Flags Updated | N, V, Z (loads asserted in S_GETC) |
| Cycles | 4 (S_GETA -> S_GETB -> S_GETC -> S_MOVREG) |

Subtracts the shifted Rm from Rn and updates the status flags, but discards the numerical result (no register write-back). The critical difference from ADD is that loads = 1 is asserted in S_GETC, which causes the flag flip-flop to capture {V, N, Z}. The result stored in C is discarded because write = 0 in S_MOVREG for CMP.

### AND Rd, Rn, Rm{,<sh_op>}  —  Bitwise AND

| Field | Value |
| --- | --- |
| Encoding | opcode = 3'b101, op = 2'b10 |
| Syntax | AND Rd, Rn, Rm{, sh_op} |
| Operation | Rd <- Rn AND shift(Rm, sh_op) |
| Flags Updated | None |
| Cycles | 4 (S_GETA -> S_GETB -> S_GETC -> S_MOVREG) |

Performs a bitwise AND of Rn and the shifted Rm, storing the result in Rd. Execution follows the same four-state sequence as ADD. The ALU receives ALUop = 2'b10 which selects the AND operation.

### MVN Rd, Rm{,<sh_op>}  —  Move Negated

| Field | Value |
| --- | --- |
| Encoding | opcode = 3'b101, op = 2'b11 |
| Syntax | MVN Rd, Rm{, sh_op} |
| Operation | Rd <- NOT shift(Rm, sh_op) |
| Flags Updated | None |
| Cycles | 4 (S_GETA -> S_GETB -> S_GETC -> S_MOVREG) (S_GETA loads Rn but Ain not used) |

Bitwise-inverts the optionally shifted value of Rm and writes it to Rd. Although the FSM passes through S_GETA, the result of the Rn read is unused because Ain is the operand that matters here. The ALU selects ALUop = 2'b11 (NOT Bin).

# 4. Internal Control Signal Reference

The FSM generates the following control signals to the datapath and instruction decoder on every clock cycle:

| Signal | Width | Active | Description |
| --- | --- | --- | --- |
| nsel | 3-bit | One-Hot High | Selects which instruction register field (Rn, Rd, Rm) is presented as readnum and writenum. 3'b001=Rn, 3'b010=Rd, 3'b100=Rm. |
| vsel | 4-bit | One-Hot High | Selects the source written into the register file. 4'b0001=C (datapath_out), 4'b0010=PC, 4'b0100=sximm8, 4'b1000=mdata. |
| loada | 1-bit | High | Loads the register file output (data_out) into flip-flop A on the next rising clock edge. |
| loadb | 1-bit | High | Loads the register file output (data_out) into flip-flop B on the next rising clock edge. |
| asel | 1-bit | High | Multiplexer select for the ALU A input. 0=flip-flop A output, 1=16'b0 (forces Ain to zero for MOV). |
| bsel | 1-bit | High | Multiplexer select for the ALU B input. 0=shifter output (sout), 1=sximm5 immediate. |
| loadc | 1-bit | High | Loads the ALU output into flip-flop C (datapath_out) on the next rising clock edge. |
| loads | 1-bit | High | Loads the ALU status flags {V, N, Z} into the status flip-flop on the next rising clock edge. |
| write | 1-bit | High | Enables a write to the register file. The target register is determined by writenum (driven by nsel). |
| w | 1-bit | High | Exported to the top level. Indicates the CPU is in the idle S_WAIT state, ready to accept a new instruction. |

# 5. FSM State Descriptions

The FSM is a synchronous Moore machine clocked on the positive edge of clk. All outputs are combinationally derived from the current state and opcode/op inputs.

| State | Encoding | Description | Next State(s) |
| --- | --- | --- | --- |
| S_WAIT | 4'b0000 | Idle state. Asserts w=1. Waits for s=1 then decodes {opcode, op} to branch to the correct execution sequence. | S_MOVim8, S_GETB, or S_GETA depending on instruction; self-loop when s=0. |
| S_MOVim8 | 4'b0001 | MOV immediate execution. Sets vsel=4'b0100, nsel=3'b001, write=1 to write sximm8 to Rn in one cycle. | S_WAIT |
| S_GETA | 4'b0100 | Fetch Rn into flip-flop A. Sets nsel=3'b001, loada=1. | S_GETB |
| S_GETB | 4'b0011 | Fetch Rm into flip-flop B. Sets nsel=3'b100, loadb=1. | S_GETC |
| S_GETC | 4'b0101 | Execute ALU operation; load result into flip-flop C. Sets loadc=1, and conditionally loads=1 for CMP. Also sets asel=1 for MOV Rd,Rm. | S_MOVREG |
| S_MOVREG | 4'b0110 | Write-back: writes C to the destination register Rd. Sets vsel=4'b0001, nsel=3'b010, write=1. | S_WAIT |

# 6. Datapath Submodule Reference

## 6.1 Register File (regfile)

Contains eight 16-bit general-purpose registers R0–R7. A 3-to-8 decoder maps writenum/readnum to individual register load enables. All registers use positive-edge-triggered D flip-flops. Simultaneous read and write to the same address causes the old value to be read in the current cycle (read-before-write behavior).

## 6.2 Barrel Shifter (shifter)

| sh_op [1:0] | Operation | Description |
| --- | --- | --- |
| 2'b00 | No shift | Pass through: sout = in |
| 2'b01 | Logical left << 1 | sout = {in[14:0], 1'b0} |
| 2'b10 | Logical right >> 1 | sout = {1'b0, in[15:1]} |
| 2'b11 | Arithmetic right >> 1 | sout = {in[15], in[15:1]} (sign bit replicated) |

## 6.3 ALU (ALU)

| ALUop [1:0] | Operation | Result | Flags |
| --- | --- | --- | --- |
| 2'b00 | ADD | Ain + Bin | Z=1 if result==0, N=result[15], V=overflow |
| 2'b01 | SUB | Ain - Bin | Z=1 if result==0, N=result[15], V=overflow |
| 2'b10 | AND | Ain & Bin | Z=1 if result==0 |
| 2'b11 | NOT | ~Bin | Z=1 if result==0 |

The flag output bus Z[2:0] carries {V, N, Z} to the status register. The status register is only updated when loads is asserted by the FSM.

## 6.4 vsel Multiplexer (four_one_mult)

| vsel | Source Selected | Notes |
| --- | --- | --- |
| 4'b0001 | datapath_out (C register) | Used during all register write-back states |
| 4'b0010 | PC (8-bit zero-padded) | Currently tied to 8'b0 at instantiation; reserved for future use |
| 4'b0100 | sximm8 | Used by MOV Rn, #imm8 |
| 4'b1000 | mdata (memory data) | Currently tied to 16'b0 at instantiation; reserved for memory load instructions |

# 7. Instruction Execution Timing

The number of clock cycles required for each instruction, counted from the cycle the FSM leaves S_WAIT until it returns to S_WAIT:

| Instruction | Clock Cycles | States Traversed |
| --- | --- | --- |
| MOV Rn, #imm8 | 1 | S_WAIT -> S_MOVim8 -> S_WAIT |
| MOV Rd, Rm{,sh_op} | 3 | S_WAIT -> S_GETB -> S_GETC -> S_MOVREG -> S_WAIT |
| ADD Rd, Rn, Rm{,sh_op} | 4 | S_WAIT -> S_GETA -> S_GETB -> S_GETC -> S_MOVREG -> S_WAIT |
| CMP Rn, Rm{,sh_op} | 4 | S_WAIT -> S_GETA -> S_GETB -> S_GETC -> S_MOVREG -> S_WAIT |
| AND Rd, Rn, Rm{,sh_op} | 4 | S_WAIT -> S_GETA -> S_GETB -> S_GETC -> S_MOVREG -> S_WAIT |
| MVN Rd, Rm{,sh_op} | 4 | S_WAIT -> S_GETA -> S_GETB -> S_GETC -> S_MOVREG -> S_WAIT |

# 8. CPU Operation Flow

To execute an instruction, the external controller must follow this sequence:

| Step | Action | Signal Condition |
| --- | --- | --- |
| 1 | Assert reset for at least one clock cycle | reset = 1 |
| 2 | Deassert reset | reset = 0 |
| 3 | Present instruction word on in[15:0] | in = <instruction> |
| 4 | Assert load for one clock cycle to latch the instruction | load = 1 |
| 5 | Deassert load | load = 0 |
| 6 | Assert s to start execution | s = 1 |
| 7 | Wait for w to return high indicating instruction completion | poll w == 1 |
| 8 | Read result from out[15:0], N, V, Z as needed | — |
| 9 | Repeat from step 3 for the next instruction | — |

# 9. Known Limitations and Design Notes

The following limitations and design notes apply to this implementation:

No branch or jump instructions: The current FSM and decoder do not implement any PC-modifying instructions. The PC input to the datapath is permanently tied to 8'b0.

No memory access instructions: The mdata input is permanently tied to 16'b0. LDR/STR instructions are not implemented.

nsel during S_WAIT: nsel is set to 3'b001 in S_WAIT regardless of the upcoming instruction. For instructions that immediately need Rm (e.g., MOV Rd, Rm), this is harmless because the relevant register is latched in S_GETB.

Flag register: Status flags are only updated when loads is asserted. Only the CMP instruction updates flags in this implementation. The N and V flag logic in the ALU is present but flag correctness for ADD overflow requires ALU enhancement.

Register file read-before-write: The register file presents the old value on data_out during the same cycle a write is performed. This is by design and does not affect correctness given the multi-cycle execution model.

One instruction at a time: The CPU has no pipeline interlock or superscalar capability. A new instruction may only be loaded after w returns high.

End of Specification — Simple 16-Bit CPU v1.0